I want to solve the economic dispatch for the Spanish electricity system and do an analysis on the results:

1.- I need the capacities of the Non-renewable sources and their operation cost as input data.

2.-For the renewable energy (wind ans solar) I will do the following:

We will take a whole year profile for both technologies, then we´ll get the P50 weekly production of a season: So, if a season has 3 months, we will sort all weeks in it and take the P50.

All the production values will be in per unit and the capacity installed will be given as an input. (we will assume the renewable cost as 0)

3.-We will assume an inelastic demand also given as an input that will depend on the season and will be chosen as well as the renewable sources (the P50 week of a season)

Solutions:

Installed capacity 2024: https://www.esios.ree.es/en/analysis/10300?vis=3&start_date=01-03-2024T00%3A00&end_date=31-03-2024T23%3A55&compare_start_date=04-01-2024T00%3A00&groupby=month&compare_end_date=03-03-2024T22%3A55

Generation (by technology) and consumption 2024:
https://www.esios.ree.es/en/analysis/10024?compare_indicators=369%2C370%2C92%2C91%2C79%2C95%2C88%2C90%2C96%2C83%2C82%2C80%2C81%2C87%2C97%2C71%2C2131%2C72%2C77%2C75%2C78%2C76%2C74%2C86%2C93%2C94%2C84%2C85%2C89%2C73%2C2143%2C2154&start_date=01-01-2024T00%3A00&geoids=&vis=1&end_date=31-12-2024T23%3A55&compare_start_date=31-12-2023T00%3A00&groupby=hour

Variable cost definition (Fuel + CO2+ O&M) (entso-e):

https://consultations.entsoe.eu/entso-e-general/proposal-for-european-resource-adequacy-assessment/supporting_documents/191205_Methodology%20for%20European%20Resource%20Adequacy%20Assessment_public%20consultation.pdf

Common structure (thermal units)

For thermal technologies (gas/oil/coal/biomass/waste if modelled as thermal):

MC=Pfuelη+EFel⋅PCO2+VOM

- Pfuel= fuel price in €/MWh_th

- η= electrical efficiency (NCV basis)

- EFel= emissions in tCO₂/MWh_e

- PCO2= €/tCO₂

- VOM= variable O&M in €/MWh_e

And when an emission factor is given per fuel energy (ENTSO-E gives kg/GJ_fuel), convert to electricity basis:

EFel [t/MWhe]=EFfuel [kg/GJ]⋅3.6η⋅1000

ENTSO-E provides η, EFfueland VOM by technology:

https://www.entsoe.eu/Documents/TYNDP%20documents/TYNDP2018/Scenarios%20Data%20Sets/Input%20Data.xlsx

CO2 price:

https://icapcarbonaction.com/en/ets/eu-emissions-trading-system-eu-ets

1) Gas (CCGT)

Equation

MCgas=Pgasηgas+EFgas,el⋅PCO2+VOMgas

Parameters (values used)

- ηgas=0.58(ENTSO-E “Gas – CCGT new”).

- EFfuel=57kgCO₂/GJ (ENTSO-E gas).

- VOMgas=1.6€/MWh (ENTSO-E “Gas – CCGT new”).

- Pgas≈35€/MWh_th (REE states “MIBGAS price for 2024 was around €35/MWh”).

https://www.sistemaelectrico-ree.es/en/spanish-electricity-system/markets/day-ahead-market

- PCO2=65.23€/tCO₂ (ICAP 2024 average secondary market price).

Computed (example)

- MCgas≈85.0€/MWh_e

2) Oil (modeled as Heavy Fuel Oil steam unit)

Equation

MCoil=Poilηoil+EFoil,el⋅PCO2+VOMoil

Parameters (values used)

- ηoil=0.40(ENTSO-E “Heavy oil – old 2”).

- EFfuel=78kgCO₂/GJ (ENTSO-E oil).

- VOMoil=3.3€/MWh (ENTSO-E heavy oil).

- Brent 2024 average =78.73$/bbl (REE).

https://www.sistemaelectrico-ree.es/en/spanish-electricity-system/markets/day-ahead-market

- USD/EUR 2024 annual avg =1.0824USD per EUR (Bundesbank, based on ECB reference rates).

- Energy per barrel: EIA gives 5,689,000 Btu per barrel of crude.

- Conversion convention: 1 MMBtu=0.293071 MWh(Platts methodology note).

- PCO2=65.23€/tCO₂.

(Those give an implied Poil≈ 43.6 €/MWh_th for this example.)

Computed (example)

- MCoil≈158.2€/MWh_e

3) Coal (Soft Coal / Lignite)

Equation

MCcoal=Pcoalηcoal+EFcoal,el⋅PCO2+VOMcoal

Parameters (values used)

- ηcoal=0.40
(Typical efficiency for lignite / soft coal plants)
Source: ENTSO-E TYNDP Input Data
https://www.entsoe.eu/Documents/TYNDP%20documents/TYNDP2018/Scenarios%20Data%20Sets/Input%20Data.xlsx

- EFfuel=101kgCO₂/GJ
(Emission factor for lignite / soft coal)
Source: ENTSO-E TYNDP Input Data
https://www.entsoe.eu/Documents/TYNDP%20documents/TYNDP2018/Scenarios%20Data%20Sets/Input%20Data.xlsx

- VOMcoal=3.3€/MWh
(Typical variable O&M for coal plants)
Source: ENTSO-E TYNDP Input Data
https://www.entsoe.eu/Documents/TYNDP%20documents/TYNDP2018/Scenarios%20Data%20Sets/Input%20Data.xlsx

- Pcoal≈12 €/MWhth
(Typical lignite/soft coal fuel price equivalent)
Source: World Bank Commodity Markets “Pink Sheet”
https://thedocs.worldbank.org/en/doc/18675f1d1639c7a34d463f59263ba0a2-0050012025/related/CMO-Pink-Sheet-January-2026.pdf

- PCO2=65.23€/tCO₂
(EU ETS 2024 average price)
Source: ICAP EU ETS
https://icapcarbonaction.com/en/ets/eu-emissions-trading-system-eu-ets

MCcoal≈92,6 €/MWh_e

4) Hydro

In pure economic dispatch models, hydro is often treated as very low marginal cost (no fuel; small variable O&M), but the real driver is water opportunity cost, which depends on reservoir constraints and is not a fixed parameter.

Simple dispatch approximation

MChydro≈0(or a small VOM)

Value used (example)

- MChydro=0€/MWh_e (typical assumption: very low variable production cost).

https://economy-finance.ec.europa.eu/system/files/2023-06/dp187_en_energy%20markets.pdf (Used also in all renewables)

5) Bio

Bioenergy plants are thermal units, so you can use the thermal equation, but fuel cost varies hugely by feedstock and contract, and ETS treatment depends on biomass definition (often zero-rated under ETS rules when qualifying as “biomass”).

If you don’t have a defensible biomass fuel-price series yet, a common simplified dispatch assumption is “near-zero marginal cost” (or fixed adder).

Simple placeholder (example)

MCbio≈0

Citing low variable cost assumption for renewables/low marginal-cost generation (as a simplification until you source biomass fuel costs).

Value used

- MCbio=0€/MWh_e

(If you want, we can do a proper bio model next: Pbiofrom pellet/biomass indices + η+ VOM.)

6) Waste

Waste-to-energy marginal cost is also site-specific (gate fees can make “fuel price” effectively negative; ETS treatment depends on fossil fraction and scope). If you don’t model that market explicitly, it’s commonly approximated as low/near-zero marginal cost in simplified dispatch.

Placeholder (example)

MCwaste≈0

(using the same “very low variable production cost” simplification family).

Value used

- MCwaste=0€/MWh_e

7) Wind

Wind has near-zero marginal cost (no fuel, very low variable cost), and typically bids at/near that level.

MCwind≈0

Value used

- MCwind=0€/MWh_e

8) Solar PV

Same logic: near-zero marginal cost; typically bid near that level (including in Iberian markets).

MCPV≈0
Value used

- MCPV=0€/MWh_e

9) Solar thermal (CSP)

CSP has no fuel (unless hybridized with gas); variable cost is usually treated as very low in simplified dispatch.

MCCSP≈0

Value used

- MCCSP=0€/MWh_e (same “low variable production cost” rationale).

10) Nuclear

Equation

MCnuclear=Fnuclear+VOMnuclear

(No fuel/efficiency term in dispatch formulation since nuclear fuel cycle cost is already expressed per MWh_e. No CO₂ cost.)

Parameters (values used)

- Fnuclear=6.5€/MWh
(Typical fuel cycle cost range 5–8 €/MWh)
Source: IEA/NEA Projected Costs of Generating Electricity 2020
https://www.iea.org/reports/projected-costs-of-generating-electricity-2020

- VOMnuclear=2€/MWh
Source: ENTSO-E TYNDP Input Data (Nuclear archetype VOM)
https://www.entsoe.eu/Documents/TYNDP%20documents/TYNDP2018/Scenarios%20Data%20Sets/Input%20Data.xlsx

- EFnuclear=0
(No combustion emissions under EU ETS)
Source: EU ETS framework
https://climate.ec.europa.eu/eu-action/eu-emissions-trading-system-eu-ets_en

Summary of computed marginal costs (example, 2024-style inputs)

- Gas (CCGT new): ~85.0 €/MWh

- Coal (Soft coal new): ~92,6 €/MWh

- Oil (Heavy oil old2): ~158.2 €/MWh

- Hydro/Bio/Waste/Wind/PV/CSP: 0 €/MWh (simplified assumption)

- Nuclear: 8,5 €/MWh